Derivation of Closed-Form Equations in simCellDT

ECM State Estimation — EKF Power Limit Simulation

# 1. Purpose and Context

The function simCellDT projects the battery cell state forward in time by DT seconds under a constant applied current iₖ, starting from the current EKF state estimate. The projection is used inside computePowerLimits to evaluate whether a candidate current magnitude violates voltage or state-of-charge (SOC) constraints at the end of the horizon.

Rather than iterating step-by-step, the function exploits the fact that each state equation is a first-order linear recurrence with constant coefficients under constant current. This allows a closed-form solution that gives the state after K = ⌈DT/δt⌉ steps in a single evaluation, without a loop.

# 2. Equivalent Circuit Model (ECM) State Equations

The cell is modelled as a 2RC Equivalent Circuit Model with hysteresis. The continuous-time state vector is:

x = [iⁱ₁,  iⁱ₂,  hₖ,  zₖ]ᵀ

where:

•  iⁱ₁ — current through the first RC branch (R₁, C₁)

•  iⁱ₂ — current through the second RC branch (R₂, C₂)

•  hₖ    — hysteresis state (−1 ≤ hₖ ≤ 1)

•  zₖ    — state of charge (SOC)

The discrete-time state equations (from the EKF implementation) are, for a timestep δt:

iⁱ₁[k+1] = a₁ · iⁱ₁[k] + (1 − a₁) · iₖ

iⁱ₂[k+1] = a₂ · iⁱ₂[k] + (1 − a₂) · iₖ

h[k+1]   = Aʰ · h[k] + (1 − Aʰ) · sgn(iₖ)

z[k+1]   = z[k] + (δt / 3600Q) · iₖ

where the per-step decay factors are:

a₁ = exp(−δt / R₁C₁)

a₂ = exp(−δt / R₂C₂)

Aʰ = exp(−|iₖ · G · δt| / 3600Q)

and G is the hysteresis gain, Q is the nominal capacity in Ah, and R₁C₁, R₂C₂ are the time constants of the two RC branches.

# 3. Closed-Form Solution for RC Branch Currents

## 3.1  General first-order linear recurrence

Each RC branch current satisfies the same structural recurrence. Consider a general sequence with constant scalar a (|a| < 1) and constant forcing term b:

x[k+1] = a · x[k] + b

Starting from x[0], unrolling gives:

x[1] = a·x[0] + b

x[2] = a·x[1] + b = a²·x[0] + ab + b = a²·x[0] + b(1 + a)

x[k] = aᵏ·x[0] + b(1 + a + a² + ⋯ + aᵏ⁻¹)

The finite geometric series in parentheses evaluates to:

∑ⱼ₌₀ᵏ⁻¹ aʲ = (1 − aᵏ) / (1 − a)    for a ≠ 1

Therefore:

x[K] = aᴿ·x[0] + b · (1 − aᴿ) / (1 − a)

## 3.2  Application to iⁱ₁ and iⁱ₂

For iⁱ₁, identify:   a = a₁,   b = (1 − a₁)·iₖ

Substituting into the general solution:

iⁱ₁[K] = a₁ᴿ·iⁱ₁[0] + (1−a₁)·iₖ · (1 − a₁ᴿ) / (1 − a₁)

The factor (1 − a₁) cancels in numerator and denominator:

⟹  iⁱ₁[K] = a₁ᴿ·iⁱ₁[0] + iₖ·(1 − a₁ᴿ)

This is the equation used in code:

ir1_f = a1k * x0[ir1Ind] + ik * (1 - a1k)

with a1k = a1 ** KDT. Identically for iⁱ₂ with a₂.

# 4. Closed-Form Solution for the Hysteresis State

## 4.1  Structure of the recurrence

The hysteresis recurrence is:

h[k+1] = Aʰ·h[k] + (1 − Aʰ)·sgn(iₖ)

This has the same form as the RC recurrence, with a = Aʰ and b = (1 − Aʰ)·sgn(iₖ). Critically, Aʰ is constant because the current iₖ is constant over the simulation horizon, so the decay factor does not change between steps.

## 4.2  Applying the general solution

Applying the result from Section 3.1 with K steps:

h[K] = Aʰᴿ·h[0] + (1−Aʰ)·sgn(iₖ) · (1 − Aʰᴿ) / (1 − Aʰ)

Cancelling (1 − Aʰ):

⟹  h[K] = Aʰᴿ·h[0] + sgn(iₖ)·(1 − Aʰᴿ)

Code implementation:

Ahk = Ah ** KDT

hk_f = Ahk * x0[hkInd] + sign_ik * (1 - Ahk)

## 4.3  Physical interpretation

The hysteresis state exponentially relaxes toward +1 during charging (sgn(iₖ) = +1) and toward −1 during discharging. The rate of relaxation depends on |iₖ|·G: higher current or higher G drives faster convergence to the steady-state hysteresis value. In the limit of many steps, h[∞] → sgn(iₖ), as expected.

# 5. SOC Integration

SOC evolves by pure Coulomb counting:

z[k+1] = z[k] + (δt / 3600Q) · iₖ

This is a simple accumulation — no feedback, no eigenvalue. After K steps:

z[K] = z[0] + K · (δt / 3600Q) · iₖ

Recognising that K·δt = sim_dt · KDT is the total simulated duration T_sim:

z[K] = z[0] + T_sim · iₖ / (3600Q)

Code:

SOC_f = x0[zkInd] + ik * sim_dt * KDT / (3600 * Q)

Note: because KDT = ⌈DT/sim_dt⌉ (ceiling), the actual simulated duration T_sim = sim_dt · KDT may exceed DT by up to one sim_dt step. This introduces a slight conservatism in the SOC prediction (a larger SOC excursion is predicted), which is acceptable for the power-limit application.

# 6. Terminal Voltage at the End of the Horizon

Once the projected state x[K] = [iⁱ₁[K], iⁱ₂[K], h[K], z[K]] has been computed, the terminal voltage is evaluated using the same output equation as iterEKF.

## 6.1  OCV with hysteresis

The open-circuit voltage blends charge and discharge OCV curves weighted by the hysteresis state:

OCV(z, h) = ((1+h)/2)·OCV_ch(z) + ((1−h)/2)·OCV_dch(z)

When h = +1 (fully charged direction), OCV = OCV_ch. When h = −1 (fully discharged direction), OCV = OCV_dch. Intermediate values interpolate linearly between the two curves.

## 6.2  Full terminal voltage

The ECM terminal voltage adds resistive drops across R₁, R₂, and the ohmic resistance R₀:

V_T = OCV(z[K], h[K]) + R₁·iⁱ₁[K] + R₂·iⁱ₂[K] + R₀·iₖ

Note that R₀ is multiplied by the applied current iₖ (not by iⁱ₁ or iⁱ₂), because R₀ is the series ohmic resistance and carries the total cell current. R₁ and R₂ carry their respective RC branch currents, which differ from the terminal current during transients.

Code:

OCV = ((1 + hk_f) / 2) * OCVch + ((1 - hk_f) / 2) * OCVdch

vDT = OCV + R1 * ir1_f + R2 * ir2_f + R0 * ik

# 7. Choice of Step Count K

The number of simulation steps is:

K = ⌈DT / sim_dt⌉

Using the ceiling ensures that the simulated interval always covers at least DT seconds. sim_dt trades off accuracy against computation. Smaller sim_dt gives a more accurate decay factor a₁ = exp(−sim_dt/R₁C₁) relative to the true continuous-time exponential, but requires a larger K. In practice, since the closed-form solution evaluates a₁ᴿ as a single power (a1 ** KDT), the cost of changing K is negligible.

For the RC state equations, regardless of K, the closed-form result is exact — the geometric-series collapse is exact arithmetic, not an approximation. The only approximation arises in the SOC ceiling rounding noted above.

# 8. Summary of Closed-Form Equations

The following equations are implemented in simCellDT:

Decay factors (per simulation step δt = sim_dt):

a₁ = exp(−sim_dt / R₁C₁),   a₂ = exp(−sim_dt / R₂C₂)

Aʰ = exp(−|iₖ · G · sim_dt| / 3600Q)

K = ⌈DT / sim_dt⌉

Projected state after K steps:

iⁱ₁[K] = a₁ᴿ · iⁱ₁[0] + iₖ · (1 − a₁ᴿ)

iⁱ₂[K] = a₂ᴿ · iⁱ₂[0] + iₖ · (1 − a₂ᴿ)

h[K]    = Aʰᴿ · h[0] + sgn(iₖ) · (1 − Aʰᴿ)

z[K]    = z[0] + sim_dt · K · iₖ / (3600Q)

Terminal voltage:

V_T = ½(1+h[K])·OCV_ch(z[K]) + ½(1−h[K])·OCV_dch(z[K]) + R₁·iⁱ₁[K] + R₂·iⁱ₂[K] + R₀·iₖ

Derivation document for simCellDT — ECM EKF Power Limit Simulation